IW1RMM CW ENCODER

Firmware v7.1.4 — Software Routine Analysis

ESP32 CYD | March 2026 — IW1RMM (Mauri)

Based on original VK2IDL firmware

# 1. Firmware Architecture

The firmware is organised into three concurrent FreeRTOS tasks. ESP32 Core 0 runs morseTask (priority 2, stack 8192 bytes) and serialTask (priority 1, stack 4096 bytes). Core 1 runs the Arduino loop() function, which manages LVGL and the graphical interface. Communication between tasks uses a displayQueue (64 messages) and an xMutex for shared variables.

# 2. Initialisation Routines

| Routine | Short description | Detail |
| --- | --- | --- |
| setup() | Firmware entry point | Starts in sequence: SD card (before display), Serial, mutex/queue, NVS preferences, GPIO pins, LVGL, touch, HW timer, graphical UI, BLE, FreeRTOS tasks. Critical order: SD must be initialised before tft.begin() to avoid SPI bus conflicts. |
| setupPins() | Configure GPIO | Sets MORSE_OUT (pin 27) as output LOW, PADDLE_DAH (22) with pull-up, PADDLE_DIT (35) as input, initialises ESP32 DAC for sidetone, activates TFT backlight (pin 21). |
| setupLVGL() | Initialise graphics | Calls lv_init(), initialises ILI9341 display via TFT_eSPI (VSPI), configures LVGL rendering buffer (320×10 pixels), registers display and touch drivers. |
| setupTouch() | Initialise XPT2046 touch | Initialises dedicated HSPI bus for touch, loads calibration values from NVS namespace 'touch_cal' (if available) or uses hardcoded defaults (x: 220-3730, y: 387-3763), enables horizontal rotation. |
| setupTimer() | 1 kHz hardware timer | Configures ESP32 hardware timer with 1 ms interrupt. Increments morseTimerTick counter used by morseDelay() for precise timing without blocking the task. |
| setupTasks() | Create FreeRTOS tasks | Launches morseTask on Core 0 (stack 8192, priority 2) and serialTask on Core 0 (stack 4096, priority 1). LVGL runs in loop() on Core 1. |
| loadPreferences() | Load NVS parameters | Reads from namespace 'cyd-morse' (NVS flash): WPM, frequency, volume, dahRatio, weight, paddleReversed, beaconInterval, contestCounter, messages msg0..msg5. Applies values and calculates Morse timing. |
| savePreferences() | Save NVS parameters | Writes all operating parameters plus 6 messages to NVS. Opens and closes the namespace to guarantee flash flush. |
| calculateMorseTiming() | Calculate Morse timing | Calculates ditLength, dahLength, elemSpace, charSpace, wordSpace in milliseconds from current WPM. If Farnsworth is active, calculates extended spacing separately. |

# 3. Main Loop and LVGL

| Routine | Short description | Detail |
| --- | --- | --- |
| loop() | Main loop Core 1 | Calls lv_timer_handler() to update LVGL GUI, processDisplayQueue() to apply messages from the display queue, sdPlayerTask() for SD player, checkSerialBle() for incoming BLE data, and updateClockLVGL() every second for the clock. |
| processDisplayQueue() | Apply display messages | Extracts messages from displayQueue produced by morseTask and updates corresponding LVGL labels (TX LED, text bars, WPM, VOL, FREQ, mode). |
| lvgl_updateTxLed() | Update TX LED | Changes the colour of the circular LED on the display: green when transmitting, dark grey when idle. |
| lvgl_appendTxBar() | Append char to text bar | Appends a character to the scrolling blue bar (L3). When L3 is full, scrolls text to rows L2 and L1. |
| lvgl_clearAll() | Clear display | Clears all three text bars and status labels on the display. |
| updateClockLVGL() | Update clock | Reads ESP32 system time and updates ui_LabelClock label every second. Label border is green if year > 1970 (RTC set), red otherwise. |
| my_disp_flush() | LVGL display flush | LVGL callback: transfers the rendered buffer to the ILI9341 display via tft.pushColors(). |
| my_touchpad_read() | LVGL touch read | LVGL callback: reads raw XPT2046 coordinates, maps them to screen coordinates 320×240 via calibration, returns pressed/released state. |

# 4. Morse Task (Core 0)

| Routine | Short description | Detail |
| --- | --- | --- |
| morseTask() | Main keyer task | Runs in a loop on Core 0. Waits 2 seconds at startup (FIX-4, avoids spurious 'E'). Then continuously calls checkPaddle() and sendFromBuffer(). Also manages automatic beacon and contest mode. |
| checkPaddle() | Read paddle state | Reads PADDLE_DIT and PADDLE_DAH pins. Based on mode (Iambic A/B, Manual, Auto) decides whether to start DIT or DAH transmission. Updates dit/dahPaddleFlag and decodes the Morse binary tree for display. |
| paddle_beginDIT() | Start DIT transmission | Activates DAC (sidetone), raises MORSE_OUT (PTT), waits DIT duration via morseDelay(), then deactivates DAC and lowers MORSE_OUT. Sends '.' symbol to display and updates decoding. |
| paddle_beginDAH() | Start DAH transmission | Same as paddle_beginDIT() but for DAH duration (ditLength × dahRatio). |
| paddle_morse_On() | Activate carrier (paddle) | Enables ESP32 DAC and raises MORSE_OUT. Used by manual key. |
| paddle_morse_Off() | Deactivate carrier (paddle) | Disables DAC and lowers MORSE_OUT. |
| paddleTimer() | Inter-element timing | Calculates timeout for current Morse character decoding. If inter-character time elapses without new elements, marks character as ready for decoding. |
| sendFromBuffer() | Transmit from buffer | Extracts characters from the buffer string and passes them to sendMorseChar(). Recognises <XX> prosign syntax. Updates text bar on display. Signals idle at end of buffer. |
| sendMorseChar(char c) | Transmit one Morse char | Converts ASCII character to DIT/DAH sequence via internal Morse table. Calls morseToneOn() and morseToneOff() for each element, respecting calculated timings. Also handles prosign characters without inter-character space. |
| morseToneOn(ms) | Activate tone for ms | Enables ESP32 DAC at set tone/volume, raises MORSE_OUT (PTT), waits ms via morseDelay(). Sends DISP_TX_LED ON to displayQueue. |
| morseToneOff(ms) | Deactivate tone for ms | Disables DAC, lowers MORSE_OUT, waits ms via morseDelay(). Sends DISP_TX_LED OFF. |
| morseDelay(ms) | Non-blocking delay | Waits ms milliseconds using morseTimerTick counter (1 kHz HW timer). Yields CPU every 1 ms with vTaskDelay(1) instead of blocking delay(). |
| onMorseTimer() | HW timer ISR | IRAM_ATTR interrupt service routine: increments morseTimerTick every millisecond. |
| addToBuffer(str) | Add text to buffer | Appends string str to the transmission buffer, adding a separator space. Thread-safe via LOCK/UNLOCK. |
| clearBuffer() | Clear buffer | Clears the transmission buffer and sends DISP_CLEAR to the display queue. |
| sendProsign(str) | Transmit prosign <XX> | Transmits prosign characters without inter-character space between them, as per international telegraph standard. |

# 5. Serial Task (Core 0)

| Routine | Short description | Detail |
| --- | --- | --- |
| serialTask() | Serial task Core 0 | Continuously reads USB serial port (Serial). In K3NG mode calls checkSerial(); in WinKey mode calls checkSerialWinkey(). Low priority (1) to avoid interfering with the keyer. |
| checkSerial() | K3NG command parser | Interprets K3NG commands arriving from serial port. Commands start with '\' followed by a letter. Handles: \W (set WPM), \F (set frequency), \Z (set time), \A-\Z (memories), \J (reset touch calibration), \Y (sidetone freq), \X (HSCW toggle), \E (extended display), and all configuration commands. Normal text is queued to the transmission buffer. |
| checkSerialWinkey() | WinKey protocol parser | Interprets the WinKey 2.x binary protocol. Handles host open (0x00), speed commands (0x02), mode register (0x0E), text (0x04), tune (0x0B), PTT timing, and all commands defined in the WK3 datasheet. |
| checkSerialBle() | BLE command parser | Processes data received in bleRxBuffer. Applies the same parser as checkSerial() or checkSerialWinkey() depending on the active mode. |
| sendStatus() | Send serial status | Responds to \S command by sending current state to serial port: WPM, freq, vol, mode, beacon, BLE, SD. |
| sendHelp() | Send serial help | Prints the list of available commands to serial port. |
| sendContestMsg() | Transmit contest message | Queues contest message with progressive counter to transmission buffer, then increments counter. |

# 6. BLE (Bluetooth Low Energy)

| Routine | Short description | Detail |
| --- | --- | --- |
| bleSetup() | Initialise BLE | Initialises BLEDevice with name 'VK2IDL_Morse', creates BLE server and Nordic UART service (NUS) with TX and RX characteristics. Registers MyServerCallbacks and MyCallbacks. |
| bleInit() | Start BLE advertising | Makes the device visible to BLE clients. Sets device name explicitly in primary advertising payload (fix v7.1.3 — resolves 'unnamed' on Android). Moves service UUID to scan response. Updates BLE button on screen. |
| bleStop() | Stop BLE | Stops advertising and disconnects clients. Releases BLE resources. |
| bleSend(str) | Send string via BLE | Sends string str to connected BLE client via TX characteristic. Does nothing if no client is connected or BLE is disabled. |
| bleSendLong(str) | Send long string via BLE | Sends long strings in chunked 20-byte packets with inter-packet delay. Used for \H, \V, \I, \P list responses. |
| updateBleButton() | Update BLE UI button | Updates BLE button text and colour in Settings tab based on state: OFF / ADV / CONN. |
| MyServerCallbacks | BLE connection callback | C++ class: onConnect() sets bleClientConnected=true and signals UI update. onDisconnect() restarts advertising if BLE is enabled. |
| MyCallbacks | BLE receive callback | C++ class: onWrite() queues received bytes in bleRxBuffer for processing in loop(). |

# 7. LVGL User Interface

| Routine | Short description | Detail |
| --- | --- | --- |
| createUI() | Create main tabview | Creates tabview with 5 tabs: Morse 1, Morse 2, SD, Settings, Spare. Calls creation function for each tab. |
| createTabMorse1() | Create Morse 1 tab | Builds main tab layout: scrolling text areas L1/L2/L3, message buttons (CQ, NAME, ANT, QTH, RST, TEST), CLEAR, STOP, TUNE, BEACON, CONTEST, MACRO, KEYER MODE buttons, WPM/VOL/FREQ controls. |
| createTabMorse2() | Create Morse 2 tab | Alternative keyer layout with same functionality but optimised arrangement for operating use. |
| createTabSD() | Create SD player tab | Builds SD panel: file/info/text labels, OPEN/PLAY/PAUSE/STOP buttons, WPM label. Updates labels with current SD card state. |
| createTabSettings() | Create Settings tab | Builds Settings panel: Ratio, Weight, Pad Reverse, Beacon, Contest, Farnsworth, SD Log, QRSS, BLE, Protocol (K3NG/WinKey), Recalibrate buttons. |
| createTabSpare() | Create Spare tab | Development tab with HSCW (High Speed CW) functionality and future controls. |
| createCommonControls() | Create common controls | Creates WPM+/-, VOL+/-, FREQ+/- controls, TX LED, Iambic Mode button — present identically in both Morse 1 and Morse 2 tabs. |
| createLogAreas() | Create scrolling text areas | Creates three LVGL labels L1, L2, L3 (Morse text bars) on a parent panel. L3 is the active blue bar (large font), L2 and L1 are black (small font). |

# 8. Button Event Handlers

| Handler | Trigger | Action |
| --- | --- | --- |
| onCQ/Name/AntRig/Qth/Rst/Test() | Tap on message button | Adds preset message to buffer. Long-press opens showMsgEditor() for editing. |
| onMacro() | Tap MACRO | Adds macroMessage to buffer. Long-press opens editor to modify macro. |
| onStop() | Tap STOP | Sets stopRequested=true. morseTask interrupts transmission at end of current element and clears buffer. |
| onClear() | Tap CLEAR | Calls clearBuffer() and sends DISP_CLEAR to display queue. |
| onTune() | Tap TUNE | Toggles TUNE mode: sends continuous carrier + PTT ON. Updates UI. |
| onBeacon() | Tap BEACON | Toggles automatic CQ message beacon transmission at set interval. |
| onContest() | Tap CONTEST | Opens Contest popup to set starting counter number. Toggles contest mode. |
| onKeyerMode() | Tap KEYER/BUFFER | Toggles between KEYER mode (transmits immediately from paddle) and BUFFER mode (accumulates in buffer before transmitting). |
| onIambicMode() | Tap iambic mode | Cycles between modes: Iambic A → Iambic B → Manual → Auto. Updates label and saves preference. |
| onWpmUp/Down() | Tap +/- WPM | Increments/decrements WPM by 1 (range 5-100). Recalculates Morse timing and updates display. |
| onVolUp/Down() | Tap +/- VOL | Changes DAC attenuation level (0=mute, 4=max). Updates display and hardware DAC. |
| onFreqUp/Down() | Tap +/- FREQ | Modifies sidetone frequency by 50 Hz (range 300-1200 Hz). Updates DAC. |
| onRatio() | Tap RATIO | Opens showPopupRatio(): slider to modify dahRatio (2.0-4.5). |
| onWeight() | Tap WEIGHT | Opens showPopupWeight(): slider to modify spacing_Weight. |
| onPadRev() | Tap PAD REV | Opens showPopupPadRev(): toggle to swap DIT/DAH on paddle. |
| onFarnsworthToggle() | Tap Farnsworth | Toggles Farnsworth. If activated, opens showPopupFarnsworth() to set spacing WPM. |
| onQrssToggle() | Tap QRSS | Cycles QRSS modes: OFF → 3s → 6s → 10s → 30s per DIT. Recalculates qrssDitMs. |
| onBleToggle() | Tap BLE | Toggles Bluetooth Low Energy. Calls bleInit() or bleStop(). |
| onProtocolToggle() | Tap K3NG/WinKey | Toggles serial protocol between K3NG (115200 baud) and WinKey 2.x (1200 baud). Updates winkeyMode and reinitialises Serial at correct baud rate. |
| onSdLogToggle() | Tap SD Log | Toggles SD logging. If activated, calls sdLogInit() to open/create log file. |
| onRecalibrate() | Tap Recalibrate | Clears touch calibration from NVS and reboots device to start guided calibration procedure. |
| showMsgEditor(idx) | Long-press on message | Opens overlay with LVGL textarea pre-loaded with current message and virtual keyboard. SAVE and CANCEL buttons close the editor. |
| onMsgEditorSave() | Tap SAVE in editor | Reads text from textarea, updates global message variable, performs NVS flush with end()/begin()/putString()/end()/begin() to guarantee persistence. |

# 9. Popups and Dialogs

| Routine | Short description | Detail |
| --- | --- | --- |
| createPopupBase(title) | Create popup structure | Creates a semi-transparent centred modal panel with title. Used as base by all popups. |
| closePopup() | Close active popup | Deletes popupBox and resets all current popup widget references. |
| showPopupRatio() | DAH Ratio popup | Shows slider to adjust dahRatio with -/+ buttons, current value label, SAVE button. |
| showPopupWeight() | Spacing Weight popup | Same as showPopupRatio() but for spacing_Weight. |
| showPopupBeacon() | Beacon Interval popup | Allows setting beacon interval in seconds (10-600). |
| showPopupContest() | Contest Counter popup | Allows setting the contest counter starting number. |
| showPopupFarnsworth() | Farnsworth WPM popup | Shows two labels (Effective WPM and Chars WPM) and slider for farnsworthWPM. Values update in real time while moving the slider. |
| showPopupHscw(wpm) | HSCW Speed popup | Slider to set HSCW (High Speed CW) speed. Called from Spare tab. |

# 10. SD Card and Player

| Routine | Short description | Detail |
| --- | --- | --- |
| sdScanFiles(target) | Scan SD for .txt files | Reads SD card root directory and populates sdFileList[] with found .txt file names (max 20). If target is specified, repositions cursor on that file. |
| sdLoadFile(filename) | Load file from SD | Opens specified .txt file, reads all content into sdText, resets sdCharIndex=0 and updates text preview labels. |
| sdOpenFilePopup() | Open file list popup | Creates popup with scrollable list of available .txt files on SD. Tap on a file calls sdLoadFile(). |
| sdCloseFilePopup() | Close file list popup | Deletes sdPopup LVGL object. |
| sdPlayerTask() | SD player task (in loop) | Called every loop() cycle. If sdState==SD_PLAYING, extracts next character from sdText and adds it to Morse transmission buffer, advancing sdCharIndex. When end of file reached, sets sdState to SD_IDLE. |
| sdUpdateFileLabel() | Update SD file label | Shows current file name and text length on SD tab. |
| sdUpdateInfoLabel() | Update SD info label | Shows current frequency and volume in SD tab. |
| sdUpdateWpmLabel() | Update SD WPM label | Updates WPM label in SD tab with current value. |
| sdUpdateButtons() | Update SD button state | Enables/disables PLAY/PAUSE/STOP buttons based on sdState (IDLE/PLAYING/PAUSED). |
| sdLogInit() | Initialise SD log | Creates or opens a CSV log file on SD with name based on uptime. Writes header with date, WPM, frequency. |
| getUptimeString() | Return uptime string | Calculates and returns time elapsed since boot in HH:MM:SS format. |

# 11. Touch Calibration (NEW v7.1.4)

| Routine | Short description | Detail |
| --- | --- | --- |
| runTouchCalibration() | Run guided calibration | Runs the 4-point interactive calibration procedure. Displays crosshair at each corner in sequence. Waits for user touch via waitTouchPoint(). Calculates and validates the 4 calibration values. Shows ACCEPT/RETRY screen. If accepted (or timeout 10s), saves to NVS namespace 'touch_cal'. |
| waitTouchPoint(x,y,label) | Wait for touch on point | Displays crosshair at coordinates (x,y) with on-screen instruction label. Waits for a touch event from XPT2046 with timeout. Serial 'R' escape cancels. Returns raw ADC coordinates. |
| loadTouchCalibration() | Load calibration from NVS | Reads cal_x_min, cal_x_max, cal_y_min, cal_y_max from 'touch_cal' NVS namespace. Validates range (x_min 100-1000, x_max 3000-4095, excursion ≥ 2000). If valid, applies to touch mapping. If invalid or absent, uses hardcoded defaults. |
| saveTouchCalibration() | Save calibration to NVS | Writes the 4 calibration values to 'touch_cal' NVS namespace with putInt(). Opens and closes namespace to guarantee flash flush. |
| resetTouchCalibration() | Reset calibration (\J) | Clears 'touch_cal' NVS namespace and triggers device reboot to restart guided calibration. Called by K3NG \J command and onRecalibrate() handler. |

# 12. Utility Functions

| Routine | Short description | Detail |
| --- | --- | --- |
| postDisplay(type, val) | Send message to display queue | Thread-safe inline: builds a DisplayMsg and sends it to displayQueue without blocking. If queue is full, message is discarded. |
| postDisplayChar(c) | Send char to text bar | Specialisation of postDisplay() for type DISP_TX_BAR: sends a single character to the blue text bar. |
| updateStatusLabels() | Update status labels | Updates WPM, VOL, FREQ labels in both Morse 1 and Morse 2 tabs. |
| getModeName() | Return mode name | Returns current keyer mode string: 'IAMB-A', 'IAMB-B', 'MANUAL', 'AUTO'. |
| calculateDitDuration() | Calculate DIT duration ms | Returns DIT duration in milliseconds at current WPM: 1200/WPM. |
| volumeUiLevel(v) | Convert volume UI level | Inline: converts internal DAC level (0-4) to screen display value (inverted: 0=mute, 4=max). |
| getQrssModeString(mode) | QRSS mode string | Returns text representation of active QRSS mode: 'OFF', '3s', '6s', '10s', '30s'. |

IW1RMM (Mauri) — IW1RMM CW Encoder — Based on VK2IDL firmware — Firmware Analysis v7.1.4 — March 2026